Diesel locomotive 2TE10

Diesel locomotive cabin 2TE10

Hello.

- Open archive tren_2te10v.rar. Do not unpack. The training apparatus can be started only from archive.

- Open a file 2TE10W.EXE. You will see the following picture:

3. To start a trip press “Enter”

4. A designation of devices:

"A" – the crane of the machinist №394

"B" – the crane of an auxiliary brake №254

"C" – a manometer. Pressure in the levelling reservoir.

"D" – a manometer. Pressure in a brake highway.

"E" – a manometer. A brake cylinder pressure.

5. It is necessary to charge a brake highway the compressed air. Move the crane №394 to the first position. The first position corresponds to position of gymnastics of a brake highway. For this purpose: the left button of a mouse press the crane №394 and move it to the first position.

6. You should notice. Pressure in the leveling reservoir and a brake highway has risen. Hoist pressure in the leveling reservoir to value 5,2 kgs/sm^2. At achievement of necessary pressure, translate the crane №394 in the second position. Press the right button of the mouse the crane №394.

7. Move the crane №254 to the first position.

8. Press the left button of the mouse the button a diesel engine. The bulb "diesel engine" should go out.

9. Move the right button of the mouse a backspacing to position forward.

10. Move a steering wheel to the first position. The locomotive will begin movement. On the screen «2» you will see value of a tractive effort on length of structure of a train. The screen №3 – a speedometer.

11. In process of increase in speed, move positions of the controller to the right. In total there are fifteen traction positions.

12. We will admit to you it is necessary to lower speed. Move the crane №394 to the fifth position. Use the right button of the mouse.

13. Reduce pressure in the levelling reservoir by size equal 0,6 kgs/sm^2. The given value corresponds to the first brake step. Stop pressure decrease in the levelling reservoir. For this purpose move the crane №394 to the fourth position.

14. The manometer №1а will show brake cylinder pressure increase. The screen «2а» will show gradual receipt of pressure in brake cylinders of your structure of a train.

15. If the braking force is insufficient, execute the second step of braking. Move the crane №394 to the fifth position. Reduce pressure in the levelling reservoir by size equal 0,6 kgs/sm^2. After achievement of necessary pressure in the levelling reservoir, move the crane №394 to the fourth position.

16. We will admit that speed has fallen to necessary value. Now it is necessary to make holiday of brakes. Move the crane №394 in the first position.

17. Hoist pressure in the levelling reservoir to value 5,8 kgs/sm^2. Also move the crane №394 to the second position.

18. The screen «2а» will show pressure decrease in brake cylinders of your train.

19. Gradually pressure in the levelling reservoir will reach values 5,2 kgs/sm^2.

20. After a train full stop, translate the crane №254 in the sixth position.